# Energy Institute AVIFF - Worked Examples : EI AVIFF Screening Report

## Line listing

| Record ID | P&ID | Line reference | Description | Notes | Pipe details | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material | Stream | Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] | % of stream | Flow Velocity [m/s] | Energy ρv² [kg/m s²] | Max. Span Length [m] | Stiffness category | LOF : Stiff | LOF : Medium-Stiff | LOF : Medium | LOF : Flexible | Flow-induced turbulence | Mechanical excitation | Pulsation Recip/PD | Pulsation from rotating stall | Pulsation from deadlegs | Pulsation from corrugations | Slugging flow | High frequency acoustic excitation | Valve transients | Cavitation and flashing | Thermowells | Small bore connections |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Figure D-1 | Stream 4 | Upstream of E401 |  | 14'' STD Carbon Steel | 14'' | STD | 355.6 | 9.525 |  | 0.00 | Carbon Steel | Stream 4(gas) | Stream 4 | gas | 0.97 | 59358 | 141.9 | 18 | 26 | .00002 | 425.7984016073446 | 1.22 | 23.22 | 100 | 10.0 | 1909 | 11 | medium-stiff | 0.01 | 0.01 | 0.03 | 0.11 | 0.01 |  |  |  | 0.00 |  |  |  |  |  |  |  |
| 2 | Figure D-1 | Stream 5 | Downstream of E401, upstream of 402 |  | 14'' STD Carbon Steel | 14'' | STD | 355.6 | 9.525 |  | 0.00 | Carbon Steel | Stream 5(gas) | Stream 5 | gas | 0.97 | 59358 | 30 | 26 | 24.5 | .00001 | 375.64219086193657 | 1.3 | 23.22 | 100 | 7.0 | 1321 | 11 | medium-stiff | 0.00 | 0.01 | 0.01 | 0.05 | 0.01 |  |  |  | 0.00 |  |  |  |  |  |  |  |
| 3 | Figure D-1 | Stream 6 | Downstream of V402, upstream of K402 |  | 14'' STD Carbon Steel | 14'' | STD | 355.6 | 9.525 |  | 0.00 | Carbon Steel | Stream 6(gas) | Stream 6 | gas | 0.97 | 53482 | 30 | 23 | 24.5 | .00001 | 394.05474816543466 | 1.34 | 21.75 | 100 | 7.0 | 1213 | 11 | medium-stiff | 0.00 | 0.01 | 0.01 | 0.05 | 0.01 | 0.40 |  |  | 0.00 |  |  |  |  |  | 0.29 |  |
| 4 | Figure D-1 | Stream 7 | Downstream of K402 |  | 8'' 120 Carbon Steel | 8'' | 120 | 219.075 | 18.263 |  | 0.00 | Carbon Steel | Stream 7(gas) | Stream 7 | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 | 100 | 9.0 | 5197 | 8 | medium-stiff | 0.01 | 0.02 | 0.04 | 0.12 | 0.02 | 0.40 |  |  | 1.00 |  |  |  |  |  |  | 0.66 |
| 5 | Figure D-1 | Recycle line (compressor discharge) | From downstream of K402 back to upstream of recycle valve |  | 8'' 120 Carbon Steel | 8'' | 120 | 219.075 | 18.263 |  | 0.00 | Carbon Steel | Stream 7(gas) | Stream 7 | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 | 100 | 9.0 | 5197 | 8 | medium-stiff | 0.01 | 0.02 | 0.04 | 0.12 | 0.02 | 0.40 |  |  | 0.00 |  |  | 0.29 |  |  |  |  |
| 6 | Figure D-1 | Recycle line (compressor suction) | From upstream of recycle valve to upstream of E401 |  | 6'' STD Carbon Steel | 6'' | STD | 168.275 | 7.112 |  | 0.00 | Carbon Steel | Recycle low pressure(gas) | Recycle low pressure | gas | 0.97 | 53482 | 141.9 | 18 | 26 | .00002 | 425.7984016073446 | 1.22 | 23.22 | 100 | 44.0 | 35294 | 7 | medium-stiff | 0.14 | 0.28 | 0.54 | 1.90 | 0.28 |  |  |  | 0.00 |  |  | 0.29 |  |  |  |  |
| 7 | Figure D-1 | Relief line (upstream of PSV) | Downstream of K402 to upstream of PSV |  | 4'' 120 Carbon Steel | 4'' | 120 | 114.3 | 11.125 |  | 0.00 | Carbon Steel | Relief high pressure(gas) | Relief high pressure | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 | 100 | 36.0 | 80379 | 6 | medium-stiff | 0.18 | 0.34 | 0.60 | 2.04 | 0.34 | 0.40 |  |  | 1.00 |  |  |  | 3.77 |  |  |  |
| 8 | Figure D-1 | Relief line (downstream of PSV) | Downstream of PSV to flare |  | 6'' STD Carbon Steel | 6'' | STD | 168.275 | 7.112 |  | 0.00 | Carbon Steel | Relief low pressure(gas) | Relief low pressure | gas | 0.97 | 53482 | 88 | 4 | 1 | .00001 | 428.4 | 1.33 | 21.75 | 100 | 199.0 | 158823 | 7 | medium-stiff | 0.43 | 0.88 | 1.71 | 6.06 | 0.88 |  |  |  | 1.00 |  |  | 1.48 | 1.77 |  |  |  |
| 9 | Figure D-3 | Stream 1 | From production header |  | 20'' STD Carbon Steel | 20'' | STD | 508 | 9.525 |  | 0.00 | Carbon Steel | Stream 1 (revised)(multiphase) | Stream 1 (revised) | multiphase | 0.8 | 855976 | 50 | 136 | 6.5 | 0 | 310.80779873363696 | 1.01 | 28.09 | 100 | 9.0 | 11791 | 9 | stiff | 0.26 | 0.63 | 1.19 | 5.47 | 0.26 |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Figure D-3 | Stream 2 | Gas to LP compressor |  | 12'' STD Carbon Steel | 12'' | STD | 323.85 | 9.525 |  | 0.00 | Carbon Steel | Stream 2 (revised)(gas) | Stream 2 (revised) | gas | 0.97 | 27180 | 50 | 5 | 6.5 | .00001 | 389.22911440640155 | 1.24 | 21.99 | 100 | 21.0 | 2141 | 12 | medium | 0.01 | 0.01 | 0.02 | 0.09 |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Figure D-3 | Stream 3 | Oil to cooler |  | 10'' STD Carbon Steel | 10'' | STD | 273.05 | 9.271 |  | 0.00 | Carbon Steel | Stream 3 (revised)(liquid) | Stream 3 (revised) | liquid | 0.02 | 303180 | 50 | 930 | 6.5 | .3935 | 275.28103012583387 | 1.08 | 38.29 | 100 | 2.0 | 2947 | 11 | medium | 0.07 | 0.15 | 0.27 | 1.12 |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Figure D-3 | Stream 4 | Produced water |  | 10'' STD Carbon Steel | 10'' | STD | 273.05 | 9.271 |  | 0.00 | Carbon Steel | Stream 4 (revised)(liquid) | Stream 4 (revised) | liquid | 0.02 | 525616 | 50 | 1000 | 6.5 | .00055 | 1293 | 1.16 | 18.05 | 100 | 3.0 | 8237 | 19 | flexible | 0.20 | 0.42 | 0.75 | 3.12 | 3.12 |  |  |  |  |  |  |  | 0.06 |  |  |  |

## Stream listing

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 4 | gas | 0.97 | 59358 | 141.9 | 18 | 26 | .00002 | 425.7984016073446 | 1.22 | 23.22 |
| Stream 5 | gas | 0.97 | 59358 | 30 | 26 | 24.5 | .00001 | 375.64219086193657 | 1.3 | 23.22 |
| Stream 6 | gas | 0.97 | 53482 | 30 | 23 | 24.5 | .00001 | 394.05474816543466 | 1.34 | 21.75 |
| Stream 7 | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 |
| Recycle low pressure | gas | 0.97 | 53482 | 141.9 | 18 | 26 | .00002 | 425.7984016073446 | 1.22 | 23.22 |
| Relief high pressure | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 |
| Relief low pressure | gas | 0.97 | 53482 | 88 | 4 | 1 | .00001 | 428.4 | 1.33 | 21.75 |
| Stream 1 (original) | multiphase | 0.8 | 461764 | 50 | 930 | 6.5 | 0 | 310.80779873363696 | 1.01 | 28.09 |
| Stream 2 (original) | gas | 0.97 | 27180 | 50 | 5 | 6.5 | .00001 | 389.22911440640155 | 1.24 | 21.99 |
| Stream 3 (original) | liquid | 0.02 | 303180 | 50 | 930 | 6.5 | .3935 | 275.28103012583387 | 1.08 | 38.29 |
| Stream 4 (original) | liquid | 0.02 | 131404 | 50 | 988 | 6.5 | .00055 | 415.52530955512975 | 1.16 | 18.05 |
| Stream 1 (revised) | multiphase | 0.8 | 855976 | 50 | 136 | 6.5 | 0 | 310.80779873363696 | 1.01 | 28.09 |
| Stream 2 (revised) | gas | 0.97 | 27180 | 50 | 5 | 6.5 | .00001 | 389.22911440640155 | 1.24 | 21.99 |
| Stream 3 (revised) | liquid | 0.02 | 303180 | 50 | 930 | 6.5 | .3935 | 275.28103012583387 | 1.08 | 38.29 |
| Stream 4 (revised) | liquid | 0.02 | 525616 | 50 | 1000 | 6.5 | .00055 | 1293 | 1.16 | 18.05 |

## Detailed listing

### Line: Stream 4

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Figure D-1 | Stream 4 | Upstream of E401 | 100 | 14'' | STD | 355.6 | 9.525 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 4 | gas | 0.97 | 59358 | 141.9 | 18 | 26 | .00002 | 425.7984016073446 | 1.22 | 23.22 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.01 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 10.84 |
| --- | --- |
| Natural Frequency [Hz] | 8.29 |
| Calculation Method | exact |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 0.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Line: Stream 5

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Figure D-1 | Stream 5 | Downstream of E401, upstream of 402 | 100 | 14'' | STD | 355.6 | 9.525 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 5 | gas | 0.97 | 59358 | 30 | 26 | 24.5 | .00001 | 375.64219086193657 | 1.3 | 23.22 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.01 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 10.84 |
| --- | --- |
| Natural Frequency [Hz] | 8.25 |
| Calculation Method | exact |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 0.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### Line: Stream 6

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Figure D-1 | Stream 6 | Downstream of V402, upstream of K402 | 100 | 14'' | STD | 355.6 | 9.525 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 6 | gas | 0.97 | 53482 | 30 | 23 | 24.5 | .00001 | 394.05474816543466 | 1.34 | 21.75 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.01 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 10.84 |
| --- | --- |
| Natural Frequency [Hz] | 8.26 |
| Calculation Method | exact |

| Mechanical Excitation (EI AVIFF Guidelines T2.3) | 0.40 |
| --- | --- |

| Equipment tag | Type | Size | Analysis Passed | Support | LOF |
| --- | --- | --- | --- | --- | --- |
|  | large motor/alternator |  |  |  | 0.40 |
|  | centrifugal compressor |  |  |  | 0.20 |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 0.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Thermowells (ASME PTC 19.3 TW - 2016) | 0.29 |
| --- | --- |

| Item tag | Type | Connection method | Root diameter (A) [mm] | Tip diameter (B) [mm] | Fillet radius at base (b) [mm] | Fillet radius at step (bs) [mm] | Internal bore diameter (d) [mm] | Unsupported length (L) [mm] | Reduced diameter length (Ls) [mm] | Minimum tip thickness (t) [mm] | Shielded length (L0) [mm] | Damping ratio [Note 1] | Velocity override (V) [m/s] [Note 2] | Material override [Note 3] | Natural frequency override [Note 4] | LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TT-001 | Tapered | Flanged/Lap-joint | 26.5 | 18 | 1 | 0 | 8 | 225 | 0 | 5 | 1 |  |  |  |  | 0.29 |

### Line: Stream 6 Thermowell TT-001 (ASME PTC 19.3-TW-2016)

| Thermowell Details. Type; Tapered. Connection; Flanged/Lap-joint. Root diameter (A); 26.5 mm. Tip diameter (B); 18 mm. Fillet radius at base (b); 1 mm. Internal bore diameter (d); 8 mm. Unsupported length (L); 225 mm. Minimum tip thickness (t); 5 mm. Shielded length (L0); 1 mm. Material; Carbon Steel. Process Data. Stream; Stream 6. Phase; gas. Operating Density; 23 kg/m³. Operating Pressure; 24.5 Bara. Operating Temperature; 30 °C. Fluid dynamic viscosity; .00001 Pa.s. Fluid velocity; 7.26 m/s. Pipe information. Nominal Bore; 14''. Schedule; STD. Outside diameter; 355.6 mm. Wall thickness; 10 mm. Internal Cladding; 0 mm. Material; Carbon Steel. | Frequency. Reynolds Number; 300600. Strouhal Number; 0.20. Scruton Number; 1.334. In Situ Resonant Frequency; 424.6 Hz. Vortex shedding Frequency; 80.6 Hz. Frequency Ratio; 0.19. Critial Velocity; 17.42 m/s. Material Properties. Max Allowable Working Stress; 138 MPa. Modulus of elasticity; 202 GPa. Reference modulus of elasticity; 203 GPa. Fatigue Stress Limit; 48.3 MPa. Material Density; 7750 kg/m³. Sensor Average Density; 2700 kg/m³. Cyclic Stresses at inline resonance. Cyclic drag stress SO,max; 200.7 MPa. Cyclic stress limit; 48.2 MPa. Steady state stresses at design velocity. Calculated Von Mises Stress; 0.5 MPa. Max. Allowable Von Mises Stress; 206.9 MPa. Dynamic stresses at design velocity. Combined Drag and Lift Stress; 0.4 MPa. Dynamic stress limit; 48.2 MPa. Pressure. Max Allowable Stem Pressure; 47.2 MPa. Max Allowable Tip Pressure; 414.4 MPa. |
| --- | --- |

| Assessment Result | PASS |
| --- | --- |

### Line: Stream 7

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | Figure D-1 | Stream 7 | Downstream of K402 | 100 | 8'' | 120 | 219.075 | 18.263 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 7 | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.02 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 8.32 |
| --- | --- |
| Natural Frequency [Hz] | 8.20 |
| Calculation Method | exact |

| Mechanical Excitation (EI AVIFF Guidelines T2.3) | 0.40 |
| --- | --- |

| Equipment tag | Type | Size | Analysis Passed | Support | LOF |
| --- | --- | --- | --- | --- | --- |
|  | large motor/alternator |  |  |  | 0.40 |
|  | centrifugal compressor |  |  |  | 0.20 |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 1.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Recycle line | 8'' | 120 | 182.55 |  | 20.2 | 5.1 |  |  | 1.00 | NaN | NaN |

| (SBC) Small bore connections (EI AVIFF Guidelines) | 0.66 |
| --- | --- |

| SBC Ref. | SBC tag | Fitting type | Length | Valves | SBC diameter | SBC location | Mainline LOF | Mainline schedule | LOF Geom/loc/mod | LOF | Walkdown comments | Survey Link |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Example D5 | Pressure tapping | weldolet | < 600mm | 1 (Less than #900) | 1'' DN25 | mid span | 1.00 | 80 | 0.66/1.00/0.66 | 0.66 |  |  |

### Line: Recycle line (compressor discharge)

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Figure D-1 | Recycle line (compressor discharge) | From downstream of K402 back to upstream of recycle valve | 100 | 8'' | 120 | 219.075 | 18.263 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 7 | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.02 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 8.32 |
| --- | --- |
| Natural Frequency [Hz] | 8.20 |
| Calculation Method | exact |

| Mechanical Excitation (EI AVIFF Guidelines T2.3) | 0.40 |
| --- | --- |

| Equipment tag | Type | Size | Analysis Passed | Support | LOF |
| --- | --- | --- | --- | --- | --- |
|  | large motor/alternator |  |  |  | 0.40 |
|  | centrifugal compressor |  |  |  | 0.20 |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 0.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| (AIV) High frequency acoustic excitation (EI AVIFF Guidelines T2.7) | 0.29 |
| --- | --- |

| Valve tag | Rated mass flow rate [kg/h] | Pressure upstream [bara] | Pressure downstream [bara] | Temperature upstream [°C] | Molecular weight [g/mol] | Low Noise trim [dB] | PWL [dB] | LOF | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Recycle | 53482 | 88 | 26 | 136.7 | 21.75 | 30 | 129.36 | 0.29 | 30dB low noise trim |

### Line: Recycle line (compressor suction)

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | Figure D-1 | Recycle line (compressor suction) | From upstream of recycle valve to upstream of E401 | 100 | 6'' | STD | 168.275 | 7.112 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Recycle low pressure | gas | 0.97 | 53482 | 141.9 | 18 | 26 | .00002 | 425.7984016073446 | 1.22 | 23.22 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.28 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 7.27 |
| --- | --- |
| Natural Frequency [Hz] | 8.62 |
| Calculation Method | exact |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 0.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| (AIV) High frequency acoustic excitation (EI AVIFF Guidelines T2.7) | 0.29 |
| --- | --- |

| Valve tag | Rated mass flow rate [kg/h] | Pressure upstream [bara] | Pressure downstream [bara] | Temperature upstream [°C] | Molecular weight [g/mol] | Low Noise trim [dB] | PWL [dB] | LOF | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Recycle | 53482 | 88 | 26 | 136.7 | 23.22 | 30 | 129.02 | 0.29 | 30dB low noise trim |

### Line: Relief line (upstream of PSV)

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Figure D-1 | Relief line (upstream of PSV) | Downstream of K402 to upstream of PSV | 100 | 4'' | 120 | 114.3 | 11.125 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relief high pressure | gas | 0.97 | 53482 | 136.7 | 62 | 88 | .00002 | 456.4 | 1.33 | 21.75 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.34 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 6.09 |
| --- | --- |
| Natural Frequency [Hz] | 7.89 |
| Calculation Method | exact |

| Mechanical Excitation (EI AVIFF Guidelines T2.3) | 0.40 |
| --- | --- |

| Equipment tag | Type | Size | Analysis Passed | Support | LOF |
| --- | --- | --- | --- | --- | --- |
|  | large motor/alternator |  |  |  | 0.40 |
|  | centrifugal compressor |  |  |  | 0.20 |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 1.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSV bypass | 2'' | 160 | 42.85 |  | 9.7 | .3 |  |  | 1.00 | NaN | NaN |

| Valve transients (EI AVIFF Guidelines T2.8) | 3.77 |
| --- | --- |

| Item tag | Item | Transient Mechanism | Upstream pressure [bara] | Downstream pressure [bara] | Upstream temperature [°C] | Is Valve downstream Of Pump? | Pump head at zero flow [bara] | Valve closure time [s] | Upstream pipe length [m] | Stiffness category | Valve Opening Fmax [kN] | Valve Closing Fmax [kN] | Valve Closing Pmax [MPa] | Valve opening LOF | Valve closing LOF | Max LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unknown |  |  |  | 136.7 |  |  |  |  |  | 6.28 | NaN | NaN | 3.77 | 0.00 | 3.77 |

### Line: Relief line (downstream of PSV)

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | Figure D-1 | Relief line (downstream of PSV) | Downstream of PSV to flare | 100 | 6'' | STD | 168.275 | 7.112 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relief low pressure | gas | 0.97 | 53482 | 88 | 4 | 1 | .00001 | 428.4 | 1.33 | 21.75 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.88 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 7.27 |
| --- | --- |
| Natural Frequency [Hz] | 8.66 |
| Calculation Method | medium-stiff |

| Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2) | 1.00 |
| --- | --- |

| Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSV bypass | 2'' | 80 | 49.25 |  | 10.2 | 1.1 |  |  | 1.00 | 5.37 | 268.35 |

| (AIV) High frequency acoustic excitation (EI AVIFF Guidelines T2.7) | 1.48 |
| --- | --- |

| Valve tag | Rated mass flow rate [kg/h] | Pressure upstream [bara] | Pressure downstream [bara] | Temperature upstream [°C] | Molecular weight [g/mol] | Low Noise trim [dB] | PWL [dB] | LOF | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relief | 53482 | 99 | 1 | 137 | 21.75 | 0 | 164.69 | 1.48 |  |

| Valve transients (EI AVIFF Guidelines T2.8) | 1.77 |
| --- | --- |

| Item tag | Item | Transient Mechanism | Upstream pressure [bara] | Downstream pressure [bara] | Upstream temperature [°C] | Is Valve downstream Of Pump? | Pump head at zero flow [bara] | Valve closure time [s] | Upstream pipe length [m] | Stiffness category | Valve Opening Fmax [kN] | Valve Closing Fmax [kN] | Valve Closing Pmax [MPa] | Valve opening LOF | Valve closing LOF | Max LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Not a valve |  |  |  | 137.7 |  |  |  |  |  | 6.29 | NaN | NaN | 1.77 | 0.00 | 1.77 |

### Line: Stream 1

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | Figure D-3 | Stream 1 | From production header | 100 | 20'' | STD | 508 | 9.525 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 1 (revised) | multiphase | 0.8 | 855976 | 50 | 136 | 6.5 | 0 | 310.80779873363696 | 1.01 | 28.09 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 0.26 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 9.03 |
| --- | --- |
| Natural Frequency [Hz] | 15.72 |
| Calculation Method | exact |

### Line: Stream 2

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Figure D-3 | Stream 2 | Gas to LP compressor | 100 | 12'' | STD | 323.85 | 9.525 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 2 (revised) | gas | 0.97 | 27180 | 50 | 5 | 6.5 | .00001 | 389.22911440640155 | 1.24 | 21.99 |

### Line: Stream 3

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11 | Figure D-3 | Stream 3 | Oil to cooler | 100 | 10'' | STD | 273.05 | 9.271 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 3 (revised) | liquid | 0.02 | 303180 | 50 | 930 | 6.5 | .3935 | 275.28103012583387 | 1.08 | 38.29 |

### Line: Stream 4

| Record Id | P&ID | Line reference | Description | % of stream | Nominal bore | Schedule | OD [mm] | WT [mm] | Cladding [mm] | Pipe pressure rating [bara] | Material |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | Figure D-3 | Stream 4 | Produced water | 100 | 10'' | STD | 273.05 | 9.271 |  | 0.00 | Carbon Steel |

| Description | Phase | Gas Void Fraction | Mass flow rate [kg/h] | Temperature [°C] | Density [kg/m³] | Static pressure [bara] | Dynamic viscosity [Pa.s] | Speed of sound [m/s] | Gamma (cp/cv) [#] | Molecular weight [g/mol] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream 4 (revised) | liquid | 0.02 | 525616 | 50 | 1000 | 6.5 | .00055 | 1293 | 1.16 | 18.05 |

| (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2) | 3.12 |
| --- | --- |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 18.61 |
| --- | --- |
| Natural Frequency [Hz] | 1.59 |
| Calculation Method | exact |

| Valve transients (EI AVIFF Guidelines T2.8) | 0.06 |
| --- | --- |

| Item tag | Item | Transient Mechanism | Upstream pressure [bara] | Downstream pressure [bara] | Upstream temperature [°C] | Is Valve downstream Of Pump? | Pump head at zero flow [bara] | Valve closure time [s] | Upstream pipe length [m] | Stiffness category | Valve Opening Fmax [kN] | Valve Closing Fmax [kN] | Valve Closing Pmax [MPa] | Valve opening LOF | Valve closing LOF | Max LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FCV1002 | Globe | Both | 6.5 | 6.5 |  | No |  | 5 | 2 |  | 0.00 | 0.23 | 0.00 | 0.00 | 0.06 | 0.06 |

## (FIT) Flow-induced turbulence (EI AVIFF Guidelines T2.2)

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 10.84 |
| --- | --- |
| Natural Frequency [Hz] | 8.29 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 10.84 |
| --- | --- |
| Natural Frequency [Hz] | 8.25 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 10.84 |
| --- | --- |
| Natural Frequency [Hz] | 8.26 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 8.32 |
| --- | --- |
| Natural Frequency [Hz] | 8.20 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 8.32 |
| --- | --- |
| Natural Frequency [Hz] | 8.20 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 7.27 |
| --- | --- |
| Natural Frequency [Hz] | 8.62 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 6.09 |
| --- | --- |
| Natural Frequency [Hz] | 7.89 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 7.27 |
| --- | --- |
| Natural Frequency [Hz] | 8.66 |
| Calculation Method | medium-stiff |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 9.03 |
| --- | --- |
| Natural Frequency [Hz] | 15.72 |
| Calculation Method | exact |

| Maximum pipe span [m](Using standard EI 2008 Steel curves) | 18.61 |
| --- | --- |
| Natural Frequency [Hz] | 1.59 |
| Calculation Method | exact |

## Mechanical Excitation (EI AVIFF Guidelines T2.3)

| Record Id | P&ID | Line reference | Description | Equipment tag | Type | Size | Analysis Passed | Support | LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Figure D-1 | Stream 6 | Downstream of V402, upstream of K402 |  | large motor/alternator |  |  |  | 0.40 |
| 3 | Figure D-1 | Stream 6 | Downstream of V402, upstream of K402 |  | centrifugal compressor |  |  |  | 0.20 |
| 4 | Figure D-1 | Stream 7 | Downstream of K402 |  | large motor/alternator |  |  |  | 0.40 |
| 4 | Figure D-1 | Stream 7 | Downstream of K402 |  | centrifugal compressor |  |  |  | 0.20 |
| 5 | Figure D-1 | Recycle line (compressor discharge) | From downstream of K402 back to upstream of recycle valve |  | large motor/alternator |  |  |  | 0.40 |
| 5 | Figure D-1 | Recycle line (compressor discharge) | From downstream of K402 back to upstream of recycle valve |  | centrifugal compressor |  |  |  | 0.20 |
| 7 | Figure D-1 | Relief line (upstream of PSV) | Downstream of K402 to upstream of PSV |  | large motor/alternator |  |  |  | 0.40 |
| 7 | Figure D-1 | Relief line (upstream of PSV) | Downstream of K402 to upstream of PSV |  | centrifugal compressor |  |  |  | 0.20 |

## Pulsation from reciprocating items (EI AVIFF Guidelines T2.4)

## Pulsation from rotating stall (EI AVIFF Guidelines T2.5)

## Flow-induced pulsation from deadlegs (EI AVIFF Guidelines T2.2)

| Record Id | P&ID | Line reference | Description | Branch ref | Branch nominal bore | Branch schedule | Branch internal diameter [mm] | Throat diameter [mm] | Throat edge radius [mm] | Deadleg length [m] | Type | Upstream straight length [m] | EI LOF | Advanced LOF (Non Structurally Resonant) | Advanced LOF (Structurally Resonant) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | Figure D-1 | Stream 7 | Downstream of K402 | Recycle line | 8'' | 120 | 182.55 |  | 20.2 | 5.1 |  |  | 1.00 | NaN | NaN |
| 7 | Figure D-1 | Relief line (upstream of PSV) | Downstream of K402 to upstream of PSV | PSV bypass | 2'' | 160 | 42.85 |  | 9.7 | .3 |  |  | 1.00 | NaN | NaN |
| 8 | Figure D-1 | Relief line (downstream of PSV) | Downstream of PSV to flare | PSV bypass | 2'' | 80 | 49.25 |  | 10.2 | 1.1 |  |  | 1.00 | 5.37 | 268.35 |

## Pulsation from corrugated pipes (EI Subsea AVIFF Guidelines E4.3)

## Slug flow (EI AVIFF Guidelines - Specialist)

## (AIV) High frequency acoustic excitation (EI AVIFF Guidelines T2.7)

| Record Id | P&ID | Line reference | Description | Valve tag | Rated mass flow rate [kg/h] | Pressure upstream [bara] | Pressure downstream [bara] | Temperature upstream [°C] | Molecular weight [g/mol] | Low Noise trim [dB] | PWL [dB] | LOF | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Figure D-1 | Recycle line (compressor discharge) | From downstream of K402 back to upstream of recycle valve | Recycle | 53482 | 88 | 26 | 136.7 | 21.75 | 30 | 129.36 | 0.29 | 30dB low noise trim |
| 6 | Figure D-1 | Recycle line (compressor suction) | From upstream of recycle valve to upstream of E401 | Recycle | 53482 | 88 | 26 | 136.7 | 23.22 | 30 | 129.02 | 0.29 | 30dB low noise trim |
| 8 | Figure D-1 | Relief line (downstream of PSV) | Downstream of PSV to flare | Relief | 53482 | 99 | 1 | 137 | 21.75 | 0 | 164.69 | 1.48 |  |

## Valve transients (EI AVIFF Guidelines T2.8)

| Record Id | P&ID | Line reference | Description | Item tag | Item | Transient Mechanism | Upstream pressure [bara] | Downstream pressure [bara] | Upstream temperature [°C] | Is Valve downstream Of Pump? | Pump head at zero flow [bara] | Valve closure time [s] | Upstream pipe length [m] | Stiffness category | Valve Opening Fmax [kN] | Valve Closing Fmax [kN] | Valve Closing Pmax [MPa] | Valve opening LOF | Valve closing LOF | Max LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | Figure D-1 | Relief line (upstream of PSV) | Downstream of K402 to upstream of PSV |  | Unknown |  |  |  | 136.7 |  |  |  |  |  | 6.28 | NaN | NaN | 3.77 | 0.00 | 3.77 |
| 8 | Figure D-1 | Relief line (downstream of PSV) | Downstream of PSV to flare |  | Not a valve |  |  |  | 137.7 |  |  |  |  |  | 6.29 | NaN | NaN | 1.77 | 0.00 | 1.77 |
| 12 | Figure D-3 | Stream 4 | Produced water | FCV1002 | Globe | Both | 6.5 | 6.5 |  | No |  | 5 | 2 |  | 0.00 | 0.23 | 0.00 | 0.00 | 0.06 | 0.06 |

## Cavitation and flashing (EI AVIFF Guidelines T2.9)

## Thermowells (ASME PTC 19.3 TW - 2016)

| Record Id | P&ID | Line reference | Description | Item tag | Type | Connection method | Root diameter (A) [mm] | Tip diameter (B) [mm] | Fillet radius at base (b) [mm] | Fillet radius at step (bs) [mm] | Internal bore diameter (d) [mm] | Unsupported length (L) [mm] | Reduced diameter length (Ls) [mm] | Minimum tip thickness (t) [mm] | Shielded length (L0) [mm] | Damping ratio [Note 1] | Velocity override (V) [m/s] [Note 2] | Material override [Note 3] | Natural frequency override [Note 4] | LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Figure D-1 | Stream 6 | Downstream of V402, upstream of K402 | TT-001 | Tapered | Flanged/Lap-joint | 26.5 | 18 | 1 | 0 | 8 | 225 | 0 | 5 | 1 |  |  |  |  | 0.29 |

### Thermowells (ASME PTC 19.3 TW - 2016) Summary Listing

| Record Id | P&ID | Line reference | Description | Item tag | Type | Connection method | In Situ Resonant Frequency | Vortex shedding Frequency | Frequency Ratio | Critial Velocity | Cyclic drag stress SO,max | Cyclic stress limit | LOF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Figure D-1 | Stream 6 | Downstream of V402, upstream of K402 | TT-001 | Tapered | Flanged/Lap-joint | 424.6 | 80.6 | 0.190 | 17.4 | 200.66 | 48.16 | 0.29 |

### Line: Stream 6 Thermowell TT-001 (ASME PTC 19.3-TW-2016)

| Thermowell Details. Type; Tapered. Connection; Flanged/Lap-joint. Root diameter (A); 26.5 mm. Tip diameter (B); 18 mm. Fillet radius at base (b); 1 mm. Internal bore diameter (d); 8 mm. Unsupported length (L); 225 mm. Minimum tip thickness (t); 5 mm. Shielded length (L0); 1 mm. Material; Carbon Steel. Process Data. Stream; Stream 6. Phase; gas. Operating Density; 23 kg/m³. Operating Pressure; 24.5 Bara. Operating Temperature; 30 °C. Fluid dynamic viscosity; .00001 Pa.s. Fluid velocity; 7.26 m/s. Pipe information. Nominal Bore; 14''. Schedule; STD. Outside diameter; 355.6 mm. Wall thickness; 10 mm. Internal Cladding; 0 mm. Material; Carbon Steel. | Frequency. Reynolds Number; 300600. Strouhal Number; 0.20. Scruton Number; 1.334. In Situ Resonant Frequency; 424.6 Hz. Vortex shedding Frequency; 80.6 Hz. Frequency Ratio; 0.19. Critial Velocity; 17.42 m/s. Material Properties. Max Allowable Working Stress; 138 MPa. Modulus of elasticity; 202 GPa. Reference modulus of elasticity; 203 GPa. Fatigue Stress Limit; 48.3 MPa. Material Density; 7750 kg/m³. Sensor Average Density; 2700 kg/m³. Cyclic Stresses at inline resonance. Cyclic drag stress SO,max; 200.7 MPa. Cyclic stress limit; 48.2 MPa. Steady state stresses at design velocity. Calculated Von Mises Stress; 0.5 MPa. Max. Allowable Von Mises Stress; 206.9 MPa. Dynamic stresses at design velocity. Combined Drag and Lift Stress; 0.4 MPa. Dynamic stress limit; 48.2 MPa. Pressure. Max Allowable Stem Pressure; 47.2 MPa. Max Allowable Tip Pressure; 414.4 MPa. |
| --- | --- |

| Assessment Result | PASS |
| --- | --- |

## (SBC) Small bore connections (EI AVIFF Guidelines)

| Record Id | P&ID | Line reference | Description | SBC Ref. | SBC tag | Fitting type | Length | Valves | SBC diameter | SBC location | Mainline LOF | Mainline schedule | LOF Geom/loc/mod | LOF | Walkdown comments | Survey Link |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | Figure D-1 | Stream 7 | Downstream of K402 | Example D5 | Pressure tapping | weldolet | < 600mm | 1 (Less than #900) | 1'' DN25 | mid span | 1.00 | 80 | 0.66/1.00/0.66 | 0.66 |  |  |